SCHEDULING AND DESPATCH CODE NO.1

GENERATION SCHEDULING

SDC1.1	INTRODUCTION

SDC1.1.1	Scheduling and Despatch Code No. 1 ("SDC1") sets out the procedure for:

(a)	the daily notification by a Generator to NIE of the Availability of any of its CDGUs and/or CD CCGT Installations in an Availability Notice;

(b)	the daily notification to NIE of whether there are any GSDPs which differ from the Contracted Generation Scheduling and Despatch Parameters in respect of the following Schedule Day by each Generator in a GSDP Notice;

(c)	in the case of a PPA CDGU and/or PPA CD CCGT Installation,  the submission to NIE of any revisions to Fuel Rates (or Generation Prices, as the case may be) for that PPA CDGU and/or PPA CD CCGT Installation by such Generator and in the case of an EC CDGU, the submission to NIE of a Supplemental Energy Bid;

(d)	the submission to NIE of Unit Nominations in relation to an EC CDGU; and

(e)	the issue by NIE of an Indicative Running Notification the day before the Schedule Day as a statement of which CDGUs and/or CD CCGT Installations may be required.

SDC1.1.2	NIE will take into account the effect of transfers across any External Interconnection when preparing its Generation Schedule (including taking into account the cost of any such transfers when compiling the Generation Schedule) and any transits across the NIE System.

SDC1.1.3	(a)	Where this SDC1 requires a Generator to submit a notice, the form of which is set out in one of the Appendices, it may instead of submitting it in writing, submit the information required in such a notice (which information must be supplied in full) by telephone (identifying unambiguously the type of notice which is thereby being submitted).

(b)	The individual who is giving the notice on behalf of the Generator must firstly specify the time at which the notice is being given, then identify himself and ask the individual receiving the notice on behalf of NIE also to identify himself. The information required by the notice shall then be given, including (without limitation) the identity of the CDGU and/or CD CCGT Installation in question.

(c)	The notice must then be confirmed by facsimile as soon as possible thereafter (and in any event be sent to NIE within 2 hours).  Where a facsimile is so sent by way of confirmation, it shall state clearly that it is in confirmation of a notice already given by telephone and must state the exact time at which the notice was given by telephone.

SDC1.1.4	In this SDC1, the term "Notice Submission Time" shall mean 1000 hours.

SDC1.1.5	For the purposes of this SDC1, any reference to the Designated Fuel for a CDGU shall, in the case of a PPA CDGU at a Range Station, be construed as a reference to the Designated Fuel for the associated Range.

SDC1.1.6	In this SDC1, where the provisions relating to CCGT Modules and CCGT Installations differ from the explicit requirements contained in a Generating Unit Agreement, a Power Station Agreement and/or a SSS Agreement, the provisions of that agreement will prevail.

SDC1.2	OBJECTIVE

To enable NIE to prepare a Generation Schedule (utilising, amongst other things, a Merit Order) to be used in the Scheduling and Despatch process and thereby to ensure (so far as possible):-

(i)	the integrity of the NIE System;

(ii)	the security and quality of supply;

(iii)	compliance with emissions requirements; and

(iv)	that there is sufficient generation capacity to meet NIE System Demand at all times together with an appropriate margin of reserve.

It is also to enable the preparation and issue of an Indicative Running Notification.

SDC1.3	SCOPE

SDC1 applies to NIE and to Generators with regard to their Generating Plant.

SDC1.4	PROCEDURE

SDC1.4.1	Availability Notice

SDC1.4.1.1	(a)	(i)	Each Generator shall, by not later than the Notice Submission Time each day, notify NIE by means of an Availability Notice:

(a)	in the case of a CDGU in the form set out in Appendix B to this SDC1;

(b)	in the case of a CD CCGT Installation, in the form set out in Appendix H to this SDC1;

(c)	or in such other form as NIE may reasonably notify to each Generator from time to time,

of the Availability of each of its CDGUs or CD CCGT Installation, as the case may be, (including, in the case of a CD CCGT Installation, its Operating Mode and the Availability of each of the CCGT Modules within it).

(ii)	The Availability Notice shall state the Availability of the relevant CDGU or CD CCGT Installation, as the case may be, including, in the case of a CD CCGT Installation, its Operating Mode and the Availability of each of the CCGT Modules within it for each period in the following Schedule Day (subject to revision under SDC1.4.2.5(a)).  In the case of a steam turbine CDGU, the MW figure stated in the Availability Notice must be a whole number and in the case of a CDGU which is an Open Cycle Gas Turbine, must be stated to one decimal place.

(b)	In the case where a PPA CDGU and/or PPA CD CCGT Installation is capable of firing on different fuels, then the Generator must submit an Availability Notice in respect of each Designated Fuel for the CDGU and/or CD CCGT Installation, each containing the information set out in (a) above for each Designated Fuel and each marked clearly to indicate for which Designated Fuel it applies. In the case where an EC CDGU is capable of firing on different fuels, then the Generator must submit an Availability Notice in respect of each fuel, on the above basis.

(c)	Not used.

(d)	(i)	In relation to CDGUs that are Open Cycle Gas Turbine Units or CD CCGT Installations (the Availability of which varies according to ambient temperature), an Availability Notice submitted by a Generator to NIE for the purposes of declaring the level of Availability of such CDGU and/or CD CCGT Installation must state the Availability at an ambient temperature of 15oC (or at such other temperature as may be stated in any Generating Unit Agreement or Power Station Agreement).

(ii)	The Availability will be deemed to be declared for all ambient temperatures by reference to the level of Contracted Capacity for a PPA CDGU and/or PPA CD CCGT Installation as specified in schedule 1 to the relevant Generating Unit Agreement adjusted by the Temperature Correction Factor set out in schedule 2 of the relevant Generating Unit Agreement by applying that absolute MW value.

(iii)	In the case of an EC CDGU, the relevant values will be set out in the Connection Agreement.

(iv)	In relation to CDGUs that are Open Cycle Gas Turbine Units or CD CCGT Installations, the term "Availability" shall be construed accordingly.

(e)	In relation to any steam turbine PPA CDGU at Kilroot Power Station, NIE may, in respect of any Settlement Period (and/or successive Settlement Periods) give notice (an "Overburn Notice") to the relevant Generator with as much notice as possible and in any event (except in the circumstances specified in (iii) below) not less than 24 hours before the start of such Settlement Period (or the first such period) with the following effect and subject as follows:

(i)	the Contracted Capacity (Coal) shall thereby be increased to Overburn Contracted Capacity in respect of such Settlement Period (or periods) following which the Generator shall redeclare the Availability of the CDGU in an Availability Notice (and, for the avoidance of doubt, such increase shall only apply for the Settlement Periods specified in the Overburn Notice);

(ii)	the aggregate number of Settlement Periods in any period of 24 hours and in any period of 12 months for which Overburn Notices may be given shall be no greater than the limits set out in paragraph 3 of schedule 1 to the relevant Generating Unit Agreement;

(iii)	NIE will waive the rebate of Availability Payments for late declaration of Availability under paragraph 13.2 of schedule 2 to the relevant Generating Unit Agreement if the Overburn Notice is issued by NIE less than 24 hours in advance of the start of the relevant Settlement Period.

(f)	Not used.

(g)	Not used.

(h)	(i)	In the case of a CD CCGT Installation, the Generator shall also submit the CD CCGT Installation Matrix (which shows the combination of CCGT Modules running in relation to any given MW output).  The CD CCGT Installation Matrix is designed to achieve certainty in knowing the number of CCGT Modules synchronised to achieve a Despatch instruction and should be consistent with the Availability data submitted under SDC1.4.1.1(a).  It should also reflect any requirements on Availabilities required by the relevant PPA, if any;

(ii)	The CD CCGT Installation Matrix can only be amended such that the CD CCGT Installation comprises different CCGT Modules if NIE gives its prior consent in writing.  Notice of the wish to amend the CCGT Modules within such a CD CCGT Installation must be given at least 12 months before it is wished for the amendment to take effect;

(iii)	In the absence of the submission of a  CD  CCGT Installation Matrix the last correctly submitted CD CCGT Installation Matrix under this SDC1.4.1.1(h) shall be taken to be the CD CCGT Installation Matrix submitted under this SDC1.4.1.1(h);

(iv)	In the case of a CD CCGT Installation Matrix submitted (or deemed to be submitted) as part of the data for CD CCGT Installations, the output of the CD CCGT Installation at any given instructed MW output must reflect the details in the CD CCGT Installation Matrix.  It is accepted that in cases of change in MW in response to Despatch instructions issued by NIE there may be a transitional variance to the conditions reflected in the CD CCGT Installation Matrix.  In achieving a Despatch instruction the range of number of CCGT Modules envisaged in moving from one MW output level to the other must not be departed from.  Each Generator shall notify NIE as soon as practicable after the event of any such variance.  It should be noted that there is a provisions in SDC1.4.2.5 for the Generator to revise the CD CCGT Installations Matrix, subject always to the provisions of SDC1.4.1.1(h);

(v)	Subject as provided above, NIE will rely on the CCGT Units specified in such Matrix running as indicated in the CD CCGT Installation Matrix when it issues a Despatch instruction in respect of the CD CCGT Installation; and

(vi)	Subject as provided in (ii) above, any changes to the CD CCGT Installation Matrix must be notified immediately to NIE in accordance with the relevant provisions of SDC1.

SDC1.4.1.2	Availability of Generating Plant

(a)	(i)	Each Generator shall subject always to the terms and conditions of the relevant Generating Unit Agreement throughout the term of the Generating Unit Agreement relating to a particular PPA CDGU and/or PPA CD CCGT Installation, maintain, repair, operate andfuel the CDGU and/or CD CCGT Installation as required by Prudent Operating Practice and any legal requirements with a view to providing the Contracted Capacity and the Contracted GSDPs, provided that in determining when so to maintain or repair the CDGU and/or CD CCGT Installation, the Generator may have regard to the amount of Availability Payments (including reductions in and rebates of Availability Payments) which may at any time be earned (or suffered) by it under the relevant Generating Unit Agreement and to its obligations under clause 5.1 of the relevant Power Station Agreement.

(ii)	Each Generator shall in relation to an EC CDGU maintain, repair, operate and fuel the CDGU as required by Prudent Operating Practice and any legal requirements with a view to providing the required System Support Services.

(b)	The Generator shall use reasonable endeavours to ensure that it does not at any time declare by issuing or allowing to remain outstanding an Availability Notice, or a GSDP Notice which declares the Availability or GSDPs of the CDGU and/or CD CCGT Installation including, in the case of a CD CCGT Installation, its Operating Mode) at levels or values different from those that the CDGU and/or CD CCGT Installation could achieve at the relevant time except:-

(i)	during periods of Planned Outage or Short Term Planned Maintenance Outage or otherwise with the consent of NIE;

(ii)	while repairing or maintaining the CDGU and/or CD CCGT Installation or equipment necessary to the operation of the CDGU and/or CD CCGT Installation where such repair or maintenance cannot reasonably, in accordance with Prudent Operating Practice, be deferred to a period of Planned Outage or Short Term Planned Maintenance Outage;

(iii)	where necessary to avoid an imminent risk of injury to persons or material damage to property (including the CDGU and/or CD CCGT Installation);

(iv)	if it is not lawful for the Generator to operate the CDGU and/or CD CCGT Installation; or

(v)	to the extent that the Generator is affected by Force Majeure under the Generating Unit Agreement;

provided that nothing in the Grid Code shall require the Generator to declare levels or values better than Contracted Capacity and Contracted GSDPs in respect of a PPA CDGU and/or PPA CD CCGT Installation.

(c)	Not used.

(d)	Not used.

SDC1.4.1.3	If at any time when the Availability of a CDGU or CD CCGT Installation is zero, an Availability Notice is given increasing the Availability of the CDGU or CD CCGT Installation with effect from a specified time, such notice shall (in the case of a Steam Turbine CDGU and a CD CCGT Installation) be construed as meaning that the CDGU is capable of being Synchronised to the NIE System at that specified time or, (in the case of CDGU which is an Open Cycle Gas Turbine), capable of being started at that specified time.  A Despatch instruction issued by NIE to Synchronise the CDGU to the NIE System or, as the case may be, start the CDGU and/or CD CCGT Installation, at or after that specified time shall be a valid Despatch instruction (regardless of the minimum time to Synchronise in the GSDPs).

SDC1.4.1.4	If at any time when a CDGU and/or CD CCGT Installation is Synchronised to the NIE System the Generator issues an Availability Notice increasing the level of Availability of the CDGU and/or CD CCGT Installation from a specified time, such notice shall be construed as meaning that the CDGU and/or CD CCGT Installation is capable of being Despatched to increase Load to levels greater than the previously prevailing level of Availability up to the level specified in the new Availability Notice but commensurate with the maximum Loading rate declared for the CDGU and/or CD CCGT Installation as a GSDP from the specified time.

SDC1.4.1.5	If at any time when a CDGU and/or CD CCGT Installation is Synchronised to the NIE System the Generator issues an Availability Notice decreasing the level of Availability of the CDGU from a specified time, such notice shall be construed as meaning that the CDGU is capable of maintaining Load at the level of the prevailing Availability until the time specified in the notice and thereafter shall be capable of maintaining Load to the level which would have been achieved if a Despatch instruction had been given to De-Load, with effect from the specified time, at the maximum De-Loading rate declared for the CDGU and/or CD CCGT Installation as a GSDP at such time down to the level of Availability specified in the new Availability Notice.

SDC1.4.2	Revisions to Standing Data

SDC1.4.2.1	GSDPs

(a)	(i)	By not later than the Notice Submission Time each day, each Generator shall in respect of each CDGU and CD CCGT Installation which has been declared Available in an Availability Notice submit to NIE in a GSDP Notice:

(a)	in the case of a CDGU, in the form set out in Appendix B to this SDC1;

(b)	in the case of a CD CCGT Installation, in the form set out in Appendix H to this SDC1; or

(dc)	or in such other form as NIE may reasonably notify to each Generator from time to time:-

(i)	in the case of a PPA CDGU and/or PPA CD CCGT Installation, showing any revisions to the GSDPs compared with the contracted GSDPs; and

(ii)	in the case of an EC CDGU, showing any revisions to the GSDPs compared with the previously submitted GSDPs (or if none has ever been submitted, the GSDPs)

to apply for the next following Schedule Day.  In the case of a CD CCGT Installation, it must also show any revisions to the GSDPs for each of the CCGT Modules within it.

(ii)	Where there are no revisions, the Generator must state in the GSDP Notice that the GSDPs are the Contracted GSDPs or the previously submitted GSDPs, as the case may be.

(b)	In the case of any revision to the GSDPs, as to which the Generator should, acting in accordance with Prudent Operating Practice, have some flexibility either in the revision itself or in the time at which the revision is to take effect NIE may, acting reasonably, suggest an amended revision (including an amendment to provide that there shall be no revision) and/or an amended time at which the revision is to take effect.  Insofar as it is able to do so without breaching any obligations regarding confidentiality contained either in the NIE Licence or in any agreement, NIE shall notify the Generator of the reasons for such request in such degree of detail as NIE considers reasonable in the circumstances.  If the Generator agrees to such suggestion (such agreement not to be unreasonably withheld) the Generator shall use reasonable endeavours to accommodate such suggestion and submit a revised GSDP Notice accordingly. In any event, NIE may require such further information on the revision as is reasonable and the Generator must give NIE such information as soon as reasonably practicable.

(c)	In the case where a CDGU is capable of firing on different fuels, then the Generator must submit a GSDP Notice in respect of each Designated Fuel for the CDGU (or fuel in the case of an EC CDGU), each containing the information set out in (a) above for each Designated Fuel (or each fuel) and each marked clearly to indicate to which Designated Fuel (or fuel) it applies.

(d)	A Generator must notify NIE as soon as it becomes aware, acting in accordance with Prudent Operating Practice, if (whether due to a defect in the CDGU and/or CD CCGT Installation or in its associated Power Station Equipment) any of its CDGUs or CD CCGT Installations is unable to meet the Spinning Reserve Capability set out (in the case of a PPA CDGU and/or PPA CD CCGT Installation) in the Sustained Load Diagram attached to schedule 8 of the relevant Generating Unit Agreement and submitted pursuant to the PC or (in the case of an EC CDGU) in the relevant System Support Services Agreement.  Such notification must be made by submitting a GSDP Notice in relation to item 2(c) of Appendix A to this SDC1.  In accordance with the Generator's obligations under SDC1.4.1.2(b), such Reserve Characteristics may only be amended (without NIE's consent) in the event of a defect in or failure of a CDGU and/or CD CCGT Installation or any associated Power Station Equipment.  Such amendment will only take effect for so long as it takes, acting in accordance with Prudent Operating Practice, for the relevant CDGU and/or CD CCGT Installation or associated Power Station Equipment or Range to be repaired and such repair must re-instate the Spinning Reserve Capability to its previous level or to such other level as NIE may, acting in accordance with Prudent Operating Practice, agree, taking into account the provisions of SDC1.4.1.2(a), and the Generator must then submit a GSDP Notice re-declaring the Spinning Reserve Capability accordingly.  The Generator must advise NIE of the nature of any such defect or failure and of the Generator's best estimate, acting as a reasonable and prudent Generator, of the time it will take to effect the repair to restore the Reserve Characteristics to their former level.

SDC1.4.2.2	Generation Other Relevant Data

(a)	By not later than the Notice Submission Time each day, each Generator shall in respect of each CDGU and/or CD CCGT Installation which has been declared Available in an Availability Notice under SDC1.4.1.1 in respect of the following Schedule Day submit to NIE in writing:

(i)	in the case of a CDGU, (in the form set out in Appendix B to this SDC1;

(ii) in the case of a CD CCGT Installation, in the form set out in Appendix H to this SDC1; and

(iii)	or in such other form as NIE may reasonably notify to each Generator from time to time),

details in relation to the following Schedule Day of any newly arisen special factors which in the reasonable opinion of the Generator may have a material effect on the likely output of such Generating Unit and/or CD CCGT Installation (including in relation to each of the CCGT Modules therein).  The notice must be consistent with the Generator's obligations under SDC1.4.1.2(b).

(b)	Where a CDGU and/or CD CCGT Installation is capable of firing on different fuels, then the Generator must submit details in respect of each Designated Fuel (or fuel in the case of an EC CDGU) for the CDGU and/or CD CCGT Installation, each containing the information set out in (a) above for each Designated Fuel and each marked clearly to indicate to which Designated Fuel (or fuel) it applies.

SDC1.4.2.3	A.	Fuel Rate

(a)	The Generation Prices for a PPA CDGU and/or PPA CD CCGT Installation in respect of the week beginning on the Schedule Day falling on the next following Monday shall be deemed to be submitted by that Generator by the Notice Submission Time each Tuesday and they shall be calculated from the relevant Energy Allowances set out in the relevant Generating Unit Agreement and the Fuel Rate(s) calculated in accordance with such Generating Unit Agreement (and, where relevant, such Power Station Agreement), unless the Generator notifies any revised Generation Prices in accordance with this SDC1.4.2.3.

(b)	By not later than the Notice Submission Time each Tuesday, each Generator may in respect of each PPA CDGU and/or PPA CD CCGT Installation submit to NIE any revision to the Generation Prices for such CDGU and/or CD CCGT Installation to apply throughout the next following week beginning on the Schedule Day falling on the next following Monday (which revision must be lower than the Generation Prices deemed to be submitted under (a) above) by specifying a lower Fuel Rate for such week in an Energy Bid Price Notice:

(i)	in the case of a CDGU, in the form set out in Appendix B to this SDC1;

(ii)	in the case of a CD CCGT Installation, in the form set out in Appendix I to this SDC1; or

(iii)	in such other form as NIE may reasonably notify to each Generator from time to time.

(c)	In so far as not revised or if the Fuel Rate in the Energy Bid Price Notice is higher than that deemed to have been submitted under (a) above, the previously submitted Generation Prices shall apply for the relevant week.

B.	Supplemental Energy Bid

(a)	Each Generator may in respect of each EC CDGU, each day prior to the Notice Submission Time submit to NIE a Supplemental Energy Bid in accordance with the Interim Settlement Code.

Excess Output

(b)	Each Generator may in respect of each  PPA CDGU and/or PPA CD CCGT Installation each day prior to the Notice Submission Time submit to NIE a set of PPA Generation Prices in respect of the following Schedule Day in relation to output above Contracted Capacity for the relevant Declared Fuel or Designated Fuel.

C.	Unit Nominations

(a)	Each Generator shall in respect of each EC CDGU and each EC CD CCGT Installation, each day prior to the Notice Submission Time submit to NIE a Unit Nomination in accordance with the Interim Settlement Code.

(b)	      Before the Notice Submission Time for each Schedule Day, each Generator must give NIE the information referred to in paragraphs (c) and (d) below  for each of its EC CDGUs and EC CD CCGT Installations.  The information must be in the form set out in Appendix G to this SDC1 or such other form as NIE may reasonably specify from time to time.  All Unit Nominations are assumed to be expressed as generator sent out energy (as that term is defined in the Interim Settlement Code) and it is the responsibility of the Generators concerned to take that into account when submitting Unit Nominations.

(c)       Each Generator must notify NIE of the number of kWh of electricity (“Local Unit Nomination”) the Generator wants to deliver onto the NIE System for supply by the Generator to Suppliers for each Settlement Period in the Schedule Day.  Local Unit Nominations must:

(i)	      if requested by NIE, specify the number of kWh of electricity being supplied to each Supplier; and

(ii)       be in whole numbers of kWh.

(d)	      Each Generator must notify NIE of the number of kWh of electricity (“Transfer Unit Nomination”) the Generator wants to deliver onto the NIE System for supply by the Generator via any External Interconnection to a transmission or distribution system located outside Northern Ireland for each Settlement Period in the Schedule Day.  Transfer Unit Nominations must be in whole numbers of kWh.

(e)	      When converted into MW at the Generator Terminals the Unit Nomination for an EC CDGU or EC CD CCGT Installation for a Settlement Period must not exceed the Availability of the EC CDGU or EC CD CCGT Installation for that Settlement Period.

(f)	      For the purposes of Despatch, NIE will apply the unit works adjustment (as that term is defined in the Interim Settlement Code) for the relevant EC CDGU or EC CD CCGT Installation to the Unit Nomination unless the relevant Connection Agreement provides for Despatch on the basis of generator sent out energy (as that term is defined in the Interim Settlement Code).

(g)	      If the Unit Nomination of an EC CDGU or EC CD CCGT Installation for a Settlement Period (also adjusted by the unit works adjustment) is less than the Minimum Generation of that EC CDGU or EC CD CCGT Installation, as the case may be, for that Settlement Period, then NIE must give a Despatch instruction corresponding to the Minimum Generation of that Generating Unit.

SDC1.4.2.4	Generating Unit Works Units

Once per week, on a day and at a time specified from time to time by NIE, each Generator must, in respect of each of its Power Stations, unless the data is supplied in some other agreed form,  submit to NIE in writing details of the Generating Unit Works Units for that Power Station consumed by that Power Station since the last submission under this SDC1.4.2.4, together with such other information as NIE may require in order to calculate the Generating Unit Works Units consumed by each CDGU and/or CD CCGT Installation at that Power Station.

SDC1.4.2.5	Further Revisions

(a)	A Generator may make revisions to the Availability Notice submitted to NIE under SDC1.4.1.1 at any time after submission of the Availability Notice in accordance with its obligations under SDC1.4.1.2(b) by submission of a revised Availability Notice which shall be:

(i)	in the case of a CDGU, in the form set out in Appendix D to this SDC1;

(ii)	in the case of a CD CCGT Installation (and the CCGT Modules therein), in the form set out in Appendix I to this SDC1; and

(iii)	or in such other form as NIE may reasonably notify to each Generator from time to time.

In the event that the Generator submits an Outage Notice under OC2 or NIE submits a Post Event Notice under OC11 in relation to any part of the period covered by the Availability Notice at any time after submission of the Availability Notice, the Generator shall be deemed to have submitted a revised Availability Notice consistent with such Outage Notice or Post Event Notice, as the case may be.  However, in the case where the Outage Notice is not issued as soon as possible and does not specify the reduced Availability, the Generator must, as soon as possible, submit a revised Availability Notice reflecting the reduced Availability.  The revisions to the Availability Notice may include revisions of the levels of Availability in the CD CCGT Installation Matrix reflecting the revised Availability.  NIE shall, insofar as it is reasonably able, take account of such revisions in the preparation of the Generation Schedule.

(b)	Not used.

(c)	If any of the data submitted to NIE under SDC1.4.2.1 and SDC1.4.2.2 changes, a Generator must (in the case of data submitted under SDC1.4.2.1 by means of a GSDP Notice) make revisions to such data and must notify NIE of any revisions to any previously revised data by submitting a revised GSDP Notice:

(i)	in the case of a CDGU, in the form set out in Appendix C to this SDC1;

(ii)	in the case of a CD CCGT Installation (and the CCGT Modules therein), in the form set out in Appendix J to this SDC1;

(iii)	or in such other form as NIE may reasonably notify to each Generator from time to time,

and must notify NIE of any Generation Other Relevant Data of which it becomes aware at any time after any original submission in writing.  NIE shall, insofar as it is reasonably able, take account of such revisions or notifications.  The provisions of SDC1.4.2.1(b) shall apply to revisions under this paragraph to data submitted under SDC1.4.2.1.

SDC1.4.2.6	(a)	(i)	Insofar as any data submitted or deemed to have been submitted on any particular day in any Availability Notice, GSDP Notice, or notice of Generation Other Relevant Data or any revision thereto is inconsistent with any other data in any other such notice, then the most recently submitted data which, if substituted for the inconsistent data, would make the data in such notices consistent, shall apply for the next following Schedule Day.

(ii)	Insofar as an Availability Notice is not submitted, the Generator shall be deemed to have submitted an Availability Notice by the Notice Submission Time stating that the Availability of the relevant CDGU or CD CCGT Installation for the whole of the following Schedule Day will be the level of Availability and Operating Mode declared in respect of the final Settlement Period of the current Schedule Day.

(iii)	Insofar as not revised, the last GSDP Notice to have been submitted shall apply for the next following Schedule Day.

(iv)	Insofar as not revised, the last notice relating to Generation Other Relevant Data to have been submitted shall apply for the next following Schedule Day.

(b)	As a general requirement, the Generator must ensure that the data in any Availability Notice, GSDP Notice, or notice of any other relevant data or any revision thereto is consistent with its obligations under SDC1.4.1.2(b).

(c)	Not used.

SDC1.4.3	Compilation of the Generation Schedule

SDC1.4.3.1	A Generation Schedule will be compiled daily by NIE as an internal statement of which CDGUs and/or CD CCGT Installations and/or transfer across any External Interconnection may be required for the next following Schedule Day.

SDC1.4.3.2	Merit Order

Subject as provided below, a  Merit Order will be compiled for each Schedule Day from the Incremental Price and the No-Load Price (which together, in relation to a particular Designated Fuel or Declared Fuel, as the case may be, shall be known as the "Merit Order Price Set") which are applicable on the basis of ascending prices so that the CDGU and/or CD CCGT Installation at the head of the Merit Order will be that CDGU and/or CD CCGT Installation which has the lowest Incremental Price per MWh and that at the foot of the Merit Order will be that CDGU and/or CD CCGT Installation which has the highest Incremental Price per MWh and taking account of average CDGU and/or CD CCGT Installation output and Generating Unit Works Units.  Once compiled, the Merit Order will be used, subject as provided in this SDC1, to determine which CDGU and/or CD CCGT Installation to Schedule and Despatch.  If there is more than one Merit Order Price Set offered, then the CDGU and/or CD CCGT Installation will appear in the Merit Order for each Merit Order Price Set submitted. No EC CDGU will be included in the Merit Order to the extent that its output is covered by a Unit Nomination.

SDC1.4.3.3	In compiling the Generation Schedule, NIE will take account of and give due weight to the following factors:

(i)	NIE System constraints from time to time, as determined by NIE;

(ii)	the level of output covered by Unit Nominations;

(iii)	in respect of CDGUs and/or CD CCGT Installations, the values of their GSDPs registered under this SDC1 and other information submitted under SDC1.4.2.2;

(iv)	the Start-Up Price of each CDGU and/or CD CCGT Installation;

(v)	the requirements, as determined by NIE for voltage control and MVAr reserves;

(vi)	CDGU and/or CD CCGT Installation stability, as determined by NIE;

(vii)	in respect of CDGUs and CD CCGT Installations, whether a low NOx burner is used;

(viii)	other matters to enable NIE to meet its Licence Standards;

(ix)	the need to provide an Operating Margin (by using the  various categories of reserve as specified in OC3), as determined by NIE;

(x)	the requirements, as determined by NIE for maintaining Frequency Control;

(xi)	Monitoring and/or Testing and/or Investigations to be carried out, or being carried out, under OC11, testing to be carried out, or being carried out, at the request of a User under OC11.8 and/or Commissioning/Acceptance Testing under the CC;

(xii)	System Tests being carried out or to be carried out under OC10;

(xiii)	any "take or pay" contract for the purchase of fuel to which a Generator is a party and the terms of which have been agreed by NIE and which impact on NIE and/or the terms of any other contract to which NIE is a party and which may, in its opinion, be relevant;

(xiv)	operation of Generating Plant over periods of low Demand to provide, in NIE's view, a sufficient Minimum Demand Regulation;

(xv)	the need for a Generator to comply with requirements in respect of emissions of SOx and of NOx in relation to a Generator's Power Station which contains PPA CDGUs and/or PPA CD CCGT Installation;

(xvi)	the inability of any CDGU and/or CD CCGT Installation to meet its full Spinning Reserve Capability; and

(xvii)	the availability and cost of transfers across any External Interconnection.

SDC1.4.3.4	Taking account of and applying the factors referred to in SDC1.4.3.3, the Generation Schedule shall be compiled by NIE to Schedule such CDGUs and/or CD CCGT Installations which have been declared Available in an Availability Notice or are the subject of a Unit Nomination and transfers across any External Interconnection:

(i)	as will ensure that all EC CDGUs are scheduled in accordance with their 	Unit Nominations;

(ii)	in accordance with the Merit Order, starting with the CDGU and/or CD CCGT Installation at the head of the Merit Order and taking into account the cost of transfers across any External Interconnection;

(iii)	as will in aggregate (after taking into account electricity delivered other than from CDGUs and CD CCGT Installations) be sufficient to match at all times (to the extent possible having regard to the Availability of CDGUs and CD CCGT Installations and to the availability of transfers across any External Interconnection) the forecast NIE Demand (derived under OC1) together with such margin of reserve as NIE shall consider to be appropriate; and

(iv)	as will in aggregate be sufficient to match minimum Demand levels together with a sufficient Minimum Demand Regulation.

The taking account of and application of the factors in SDC1.4.3.3 will mean that, in general, the strict Merit Order may not necessarily be followed.

SDC1.4.3.5	After the completion of the Scheduling process, but before the issue of Indicative Running Notifications, NIE may consider it necessary to make adjustments to the output of the Scheduling process.  Such adjustments could be made necessary by any of the following factors:

(a)	changes to Availability and/or GSDPs of CDGUs and CD CCGT Installations, notified to NIE after the commencement of the Scheduling process;

(b)	changes to NIE Demand forecast(s);

(c)	changes to NIE System constraints, emerging from the necessarily iterative process of Scheduling and network security assessment;

(d)	changes to CDGU and CD CCGT Installation requirements following notification to NIE of the changes in capability of a Generator to provide a Special Action as described in SDC2;

(e)	changes to CDGU and CD CCGT Installation requirements within constrained groups, following re-appraisal of NIE Demand forecasts within that constrained group;

(f)	changes to any conditions which in the reasonable opinion of NIE, would impose increased risk to the NIE System and would therefore require NIE to increase Operating Margin.  Examples of these conditions are:

(i)	unpredicted NIE System Outages which places (say) more than the equivalent of one large CDGU and/or CD CCGT Installation at risk to a secured fault;

(ii)	unpredicted Outage of Generating Plant equipment which imposes increased risk to the station output;

(iii)	volatile weather giving rise to low confidence in NIE Demand forecasts;

(iv)	severe weather conditions imposing high risk to the Total System;  and

(g)	known (or emerging) limitations and/or deficiencies of the NIE Scheduling process.

SDC1.4.3.6	When:

(a)	adverse weather is anticipated;

(b)	there is a high risk to the whole or part of the NIE System;

(c)	Demand Control has been instructed by NIE; or

(d)	a Total or Partial Shutdown exists;

these factors may mean that a CDGU or CD CCGT Installation is chosen other than in accordance with the Merit Order to a greater degree than would be the case when merely taking into account and giving due weight to the factors listed in SDC1.4.3.3 in order to seek to maintain the integrity of the NIE System.

SDC1.4.3.7	(a)	The Synchronising and De-Synchronising times shown in the Indicative Running Notification are indicative only and it should be borne in mind by Generators that the Despatch instructions could reflect more or different CDGU or CD CCGT Installation requirements than in the Indicative Running Notification, although they will (subject to SDC2) reflect Unit Nominations.  NIE may issue Despatch instructions in respect of any CDGU or CD CCGT Installation which has not been declared wholly Unavailable in an Availability Notice.  Generators with CDGUs and/or CD CCGT Installations must ensure that their Generating Units are able to be Synchronised at the times Scheduled but only if so Despatched by NIE by issue of a Despatch instruction.  Generators shall, as part of a revision to the GSDPs, indicate to NIE the latest time at which a Despatch instruction is required to meet the Scheduled Synchronising time.

(b)	Not used.

SDC1.4.3.8	Content of Indicative Running Notification

The information contained in the Indicative Running Notification will indicate, on an individual CDGU or CD CCGT Installation basis, the period, Loading and Declared Fuel for which it is Scheduled during the following Schedule Day.  In the case of a CDGU or CD CCGT Installation which is capable of firing on two different Designated Fuels, it will also indicate the Designated Fuel for which it is Scheduled during the following Schedule Day.  If no Declared Fuel and/or, where relevant, Designated Fuel is contained in the Indicative Running Notification, then the most  recently specified Declared Fuel and/or, where relevant, Designated Fuel shall be treated as having been indicated.   Equivalent rules apply to fuels at EC CDGUs.

SDC1.4.3.9	Issue of Indicative Running Notification

(a)	The Indicative Running Notification will be issued in writing to Generators by 1800 hours each day, provided that all the necessary information from the Generators was made available by not later than the Notice Submission Time.  However, if on any occasion NIE is unable to meet this time, NIE also reserves the right to extend the timescale for Indicative Running Notification issue to the extent necessary.

(b)	NIE may issue Despatch instructions to CDGUs and/or CD CCGT Installations before the issue of the Indicative Running Notification for the Schedule Day to which the Despatch instruction relates if the length of Notice to Synchronise the relevant CDGUs and/or CD CCGT Installations requires the Despatch instruction to be given at that time.  When the length of the time required for Notice to Synchronise is within 30  minutes of causing the CDGU and/or CD CCGT Installation to be unable to meet the indicative Synchronising time in the Indicative Running Notification or a subsequent indicative Synchronising time and no Despatch instruction has been received, the Generator must inform NIE without delay.

SDC1.4.3.10	The Indicative Running Notification received by each Generator will contain information relating to its CDGUs and CD CCGT Installations only.

SDC1.4.3.11	Minimum Demand Regulation (MDR)

It is a requirement for running the NIE System that all Synchronised CDGUs and CD CCGT Installations must at all times be capable of reducing output sufficient to allow a sufficient regulating margin for adequate Frequency Control.  NIE will monitor the output data of the Generation Schedule against forecast NIE Demand to see whether the level of MDR for any period is insufficient, and may take any shortfall into account in Scheduling and Despatch.

SDC1.4.3.12	Data Requirements

Appendix A sets out the GSDPs for which values are to be supplied by a Generator in respect of each of its CDGUs and/or CD CCGT Installations by not later than the Notice Submission Time on the day prior to the Schedule Day.

SDC1 - APPENDIX A

Generation Scheduling and Despatch Parameters

A.	For each CDGU or CD CCGT Installation:-

1.	(a)	in the case of steam turbine CDGUs, Synchronising times for the various levels of Warmth;

(b)	in the case of CDGUs which are Open Cycle Gas Turbines and CD CCGT Installations, the time from initiation of a start to achieving Despatched Load.

2.	basic data:

(a)	Minimum Generation in MW;

(b)	Governor Droop (%);

(c)	Sustained Response Capability.

3.	two shifting limitation (limitation on the number of Start-ups per Schedule Day);

4.	minimum on time;

5.	block Load following Synchronisation in MW;

6.	maximum Loading rates for the various levels of Warmth and for up to two output ranges and, in the case of CDGUs at Coolkeeragh Power Station only, soak times for the various levels of Warmth;

7.	maximum De-Loading rates for up to two output ranges;

8.	the MW and MVAr capability limits within which the CDGU or CD CCGT Installation is able to operate as shown in the relevant Generator Performance Chart;

9.	maximum number of changes to the Despatched Fuel per 24 hour period;

10.	maximum quantity of oil in "ready-use tank(s)" and associated pipework;

11.	maximum number of changes to the Designated Fuel per 24 hour period;

12.	minimum notice to change the Designated Fuel;

13.	maximum number of on Load cycles per 24 hour period, together with the maximum Load increases involved; and

14.	settings of the Unit Load Controller for each CDGU for which a Unit Load Controller is required under CCS1.5.5.

15.	in the case of CDGUs which are Open Cycle Gas Turbines and CD CCGT Installations only, the Declared Peak Capacity.

In the case of a Gas Turbine Unit, only the data applicable to Gas Turbine Units should be supplied.

B.	For the Power Station of which the CDGU or CD CCGT Installation forms part:-

1.	time between Synchronising different CDGUs and/or CD CCGT Installations in a Power Station taking account of actual off-Load periods for the various levels of Warmth and in the case of Coolkeeragh Power Station only, for both paired and single CDGUs; and

2.	time between De-Synchronising different CDGUs and/or CD CCGT Installations in a Power Station which, in the case of Coolkeeragh Power Station only, shall be stated for both paired and single CDGUs.

C.	For each CCGT Module:-

The equivalent information as set out in paragraph A above for CD CCGT Installations is to be provided for each CCGT Module within every CD CCGT Installation.

@@

SDC1 - Appendix B

FACSIMILE

TO	:	Northern Ireland Electricity plc,  Castlereagh House Control Centre             Page 1 of

FAX	:

FROM	:

[               ] POWER STATION - DAILY SCHEDULING NOTICES

_____________________________________________________________________________________________________________________

1.	AVAILABILITY NOTICE

The availability of each CDGU at [             ] Power Station for the Schedule Day to which this notice refers is shown on page 2 attached.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

2.	ENERGY BID PRICE NOTICE   (£x.xxxx/GJ - enter bid price where appropriate)

[           Power Limited] offers NIE the following energy prices (which are lower than the contracted prices) for the Schedule Day to which this notice refers.

[Fuel]				[Fuel]

[Fuel]				[Fuel]

[Fuel]

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

3.	GSDP NOTICE

The Generation Scheduling and Despatch Parameters for each CDGU at [       ] Power Station for the Schedule Day to which this notice refers are as given in schedule 1 of the relevant Generating Unit Agreement

YES / NO  (Delete as appropriate)

(If NO, each GSDP which varies from schedule 1 must be shown on page 3 attached.

If YES, page 3 shall be omitted)

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

4.	GENERATION OTHER RELEVANT DATA

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

5.	DECLARATION

These notices are issued by [           Power Limited] in respect of the Centrally Despatched Generating Units (CDGUs)

at [             ] Power Station in accordance with the requirements of SDC1.

We declare that the figures and other information given in the notices fully reflect the actual capability of the CDGUs.

The notices are applicable to Schedule Day:

_______________________________________________________________________________________________________________

Signature:________________________Name:___________________________________________________________________________

(in block capitals)

Position: Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue:

________________________________________________________________________________________________________________

AVAILABILITY NOTICE				Page 2 of @@

| Settlement Period | [UNITS ON OIL] |  | [UNITS ON COAL] |  | GAS TURBINES REDUCTION |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contracted Capacity | 1 | 2 | 1 | 2 | 1 | 2 |
| 0000 - 0030 _____ _____ _____ _____ _____ _____ 0030 - 0100 _____ _____ _____ _____ _____ _____ 0100 - 0130 _____ _____ _____ _____ _____ _____ 0130 - 0200 _____ _____ _____ _____ _____ _____ 0200 - 0230 _____ _____ _____ _____ _____ _____ 0230 - 0300 _____ _____ _____ _____ _____ _____ 0300 - 0330 _____ _____ _____ _____ _____ _____ 0330 - 0400 _____ _____ _____ _____ _____ _____ 0400 - 0430 _____ _____ _____ _____ _____ _____ 0430 - 0500 _____ _____ _____ _____ _____ _____ 0500 - 0530 _____ _____ _____ _____ _____ _____ 0530 - 0600 _____ _____ _____ _____ _____ _____ 0600 - 0630 _____ _____ _____ _____ _____ _____ 0630 - 0700 _____ _____ _____ _____ _____ _____ 0700 - 0730 _____ _____ _____ _____ _____ _____ 0730 - 0800 _____ _____ _____ _____ _____ _____ 0800 - 0830 _____ _____ _____ _____ _____ _____ 0830 - 0900 _____ _____ _____ _____ _____ _____ 0900 - 0930 _____ _____ _____ _____ _____ _____ 0930 - 1000 _____ _____ _____ _____ _____ _____ 1000 - 1030 _____ _____ _____ _____ _____ _____ 1030 - 1100 _____ _____ _____ _____ _____ _____ 1100 - 1130 _____ _____ _____ _____ _____ _____ 1130 - 1200 _____ _____ _____ _____ _____ _____ 1200 - 1230 _____ _____ _____ _____ _____ _____ 1230 - 1300 _____ _____ _____ _____ _____ _____ 1300 - 1330 _____ _____ _____ _____ _____ _____ 1330 - 1400 _____ _____ _____ _____ _____ _____ 1400 - 1430 _____ _____ _____ _____ _____ _____ 1430 - 1500 _____ _____ _____ _____ _____ _____ 1500 - 1530 _____ _____ _____ _____ _____ _____ 1530 - 1600 _____ _____ _____ _____ _____ _____ 1600 - 1630 _____ _____ _____ _____ _____ _____ 1630 - 1700 _____ _____ _____ _____ _____ _____ 1700 - 1730 _____ _____ _____ _____ _____ _____ 1730 - 1800 _____ _____ _____ _____ _____ _____ 1800 - 1830 _____ _____ _____ _____ _____ _____ 1830 - 1900 _____ _____ _____ _____ _____ _____ 1900 - 1930 _____ _____ _____ _____ _____ _____ 1930 - 2000 _____ _____ _____ _____ _____ _____ 2000 - 2030 _____ _____ _____ _____ _____ _____ 2030 - 2100 _____ _____ _____ _____ _____ _____ 2100 - 2130 _____ _____ _____ _____ _____ _____ 2130 - 2200 _____ _____ _____ _____ _____ _____ 2200 - 2230 _____ _____ _____ _____ _____ _____ 2230 - 2300 _____ _____ _____ _____ _____ _____ 2300 - 2330 _____ _____ _____ _____ _____ _____ 2330 - 2400 _____ _____ _____ _____ _____ _____ |  |  |  |  |  |  |

@@

_____________________________________________________________________________________________________________________

Notes

1.	All availabilities shall be expressed in MW.  Alongside the MW figure shall be placed the unit operating status (in brackets) as defined by the Final Outage Programme: 1 = Normal Availability, 2 = Inflexible Outage, 3 = Flexible Planned Outage, 4 = Advanced Flexible Planned Outage, 5 = Deferred Flexible Planned Outage, 6 = Short Term Planned Maintenance Outage, 7 = Return to Service, 8 = Notified Unplanned Outage, 9 = Forced Outage.

2.	For each CDGU an availability figure must be entered for the first Settlement Period.  Where the CDGU is completely unavailable, a zero shall be entered.  Thereafter, an availability figure shall only be entered where the availability for the CDGU is changed from the previously expressed value.

3.	This Availability Notice shall include all planned outages agreed with NIE and all unplanned/forced outages already notified to NIE.  It shall not include unplanned/forced outages not yet notified to NIE unless the appropriate Outage Notice is attached.

_____________________________________________________________________________________________________________________

This notice is applicable to Schedule Day: ________________________________________________

Signature: ___________________________________________________

_____________________________________________________________________________________________________________________

GSDP NOTICE				Page 3 of

_____________________________________________________________________________________________________________________

[         Power Limited] declares that the undermentioned CDGUs are presently unable to perform to the characteristics stated in [schedule 1 of the relevant Generating Unit Agreement] and that the affected characteristics are mentioned below with revised values that should be used for the purposes of scheduling and despatch.

GENERATION SCHEDULING AND 	CONTRACTED	REVISED	      TIME

UNIT	DESPATCH PARAMETER AFFECTED	VALUE	VALUE	FROM	TO

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

Examples

1.	Governor Droop	4.0%	4.5%	0000	2400

2.	Loading Rate after Hot Start 30+ MW	6.0	2.5	1800	2200

_____________________________________________________________________________________________________________________

This notice is applicable to Schedule Day: ___________________________________________________________

Signature: __________________________________________________

_____________________________________________________________________________________________________________________

SDC1 - Appendix C

FACSIMILE				Page 1 of

TO/FROM:	Northern Ireland Electricity plc, Castlereagh House Control Centre

FAX:		[		]

FROM/TO:	[                   Power Limited]

GSDP REVISION NOTICE

________________________________________________________________________________________________________

1.	[          ] POWER STATION REQUEST

[       Power Limited] advises NIE that the undermentioned GSDPs for the following CDGUs are not in accordance with

schedule 1 of the relevant GUAs and, as a result, the CDGUs will be unable to meet their Spinning Reserve Capability:

ESTIMATED

CONTRACTED	REVISED	      START		DURATION

UNIT ID	GSDP AFFECTED	  VALUE   	 VALUE 	DATE	TIME	   HRS

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

Reason for Change

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

Signature: _______________________________________	Name: ________________________________________

(in block capitals)

Position: Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue: ______________________________________

________________________________________________________________________________________________________

2.	NIE RESPONSE TO REQUEST (Delete as applicable)

1.  Request accepted and confirmed as above.

2.  Amendment proposed as shown below:

REVISED VALUE:  _____________________    REVISED ESTIMATED DURATION HRS: _________________

REVISED START DATE: _________________    REVISED START TIME:   ___________________________

Signature: ________________________________    Name: ___________________________________________

(in block capitals)

Position:  Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue: __________________________________

________________________________________________________________________________________________________

SDC1 - Appendix D

NOTIFICATION OF REVISED AVAILABILITY @@

| Settlement Period | UNITS ON OIL 1 2 |  | UNITS ON COAL 1 2 |  | GAS TURBINES 1 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contracted Capacity |  |  |  |  |  |  |
| 0000 - 0030 _____ _____ _____ _____ _____ _____ 0030 - 0100 _____ _____ _____ _____ _____ _____ 0100 - 0130 _____ _____ _____ _____ _____ _____ 0130 - 0200 _____ _____ _____ _____ _____ _____ 0200 - 0230 _____ _____ _____ _____ _____ _____ 0230 - 0300 _____ _____ _____ _____ _____ _____ 0300 - 0330 _____ _____ _____ _____ _____ _____ 0330 - 0400 _____ _____ _____ _____ _____ _____ 0400 - 0430 _____ _____ _____ _____ _____ _____ 0430 - 0500 _____ _____ _____ _____ _____ _____ 0500 - 0530 _____ _____ _____ _____ _____ _____ 0530 - 0600 _____ _____ _____ _____ _____ _____ 0600 - 0630 _____ _____ _____ _____ _____ _____ 0630 - 0700 _____ _____ _____ _____ _____ _____ 0700 - 0730 _____ _____ _____ _____ _____ _____ 0730 - 0800 _____ _____ _____ _____ _____ _____ 0800 - 0830 _____ _____ _____ _____ _____ _____ 0830 - 0900 _____ _____ _____ _____ _____ _____ 0900 - 0930 _____ _____ _____ _____ _____ _____ 0930 - 1000 _____ _____ _____ _____ _____ _____ 1000 - 1030 _____ _____ _____ _____ _____ _____ 1030 - 1100 _____ _____ _____ _____ _____ _____ 1100 - 1130 _____ _____ _____ _____ _____ _____ 1130 - 1200 _____ _____ _____ _____ _____ _____ 1200 - 1230 _____ _____ _____ _____ _____ _____ 1230 - 1300 _____ _____ _____ _____ _____ _____ 1300 - 1330 _____ _____ _____ _____ _____ _____ 1330 - 1400 _____ _____ _____ _____ _____ _____ 1400 - 1430 _____ _____ _____ _____ _____ _____ 1430 - 1500 _____ _____ _____ _____ _____ _____ 1500 - 1530 _____ _____ _____ _____ _____ _____ 1530 - 1600 _____ _____ _____ _____ _____ _____ 1600 - 1630 _____ _____ _____ _____ _____ _____ 1630 - 1700 _____ _____ _____ _____ _____ _____ 1700 - 1730 _____ _____ _____ _____ _____ _____ 1730 - 1800 _____ _____ _____ _____ _____ _____ 1800 - 1830 _____ _____ _____ _____ _____ _____ 1830 - 1900 _____ _____ _____ _____ _____ _____ 1900 - 1930 _____ _____ _____ _____ _____ _____ 1930 - 2000 _____ _____ _____ _____ _____ _____ 2000 - 2030 _____ _____ _____ _____ _____ _____ 2030 - 2100 _____ _____ _____ _____ _____ _____ 2100 - 2130 _____ _____ _____ _____ _____ _____ 2130 - 2200 _____ _____ _____ _____ _____ _____ 2200 - 2230 _____ _____ _____ _____ _____ _____ 2230 - 2300 _____ _____ _____ _____ _____ _____ 2300 - 2330 _____ _____ _____ _____ _____ _____ 2330 - 2400 _____ _____ _____ _____ _____ _____ |  |  |  |  |  |  |

@@____________________________________________________________________________________________________________

Notes

1.	All Availabilities shall be expressed in MW.  Alongside the MW figure shall be placed the unit operating status (in brackets) as defined by the Final Outage Programme: 1 = Normal Availability, 2 = Inflexible Planned Outage, 3 = Flexible Planned Outage, 4 = Advanced Flexible Planned Outage, 5 = Deferred Flexible Planned Outage, 6 = Short Term Planned Maintenance Outage, 7 = Return to Service.

2.	For each CDGU, an Availability figure shall be entered in respect of the relevant Settlement Period(s) where the Availability of the CDGU is changed from a value notified previously in an Availability Notice. Where the change in Availability occurs otherwise than at the commencement of an Availability Period, an appropriate annotation must be made, stating the exact time of the change.  If the CDGU is Unavailable, a zero shall be entered.

_______________________________________________________________________________________________________

This notice is applicable to Schedule Day: ____________________________________________________________________

Signature: ___________________________________________________

SDC1 - APPENDIX E

[Not  used]

SDC1 - APPENDIX F

Not used.

# Appendix G

TRADING DATA: PARTICIPATING GENERATORS

PART 1: LOCAL UNIT NOMINATIONS

## Name of Participating Generator

## Name of Generating Unit

## Date on which Trading Day commences

## Local Unit Nomination (SDC1.4.2.3C (c))@@

| Settlement Period |  |  | Supplier Allocation (see 5 below) |  |  |  |  | Local Unit Nomination |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PS1 |  | PS2 | PS3 | (etc) | (Totals) |
| Start | End |  | (kWh) |  | (kWh) | (kWh) |  | (kWh) |
| 06.00 | 06.30 |  |  |  |  |  |  |  |
| 06.30 | 07.00 |  |  |  |  |  |  |  |
| 07.00 | 07.30 |  |  |  |  |  |  |  |
| 07.30 | 08.00 |  |  |  |  |  |  |  |
| 08.00 | 08.30 |  |  |  |  |  |  |  |
| 08.30 | 09.00 |  |  |  |  |  |  |  |
| 09.00 | 09.30 |  |  |  |  |  |  |  |
| 09.30 | 10.00 |  |  |  |  |  |  |  |
| 10.00 | 10.30 |  |  |  |  |  |  |  |
| 10.30 | 11.00 |  |  |  |  |  |  |  |
| 11.00 | 11.30 |  |  |  |  |  |  |  |
| 11.30 | 12.00 |  |  |  |  |  |  |  |
| 12.00 | 12.30 |  |  |  |  |  |  |  |
| 12.30 | 13.00 |  |  |  |  |  |  |  |
| 13.00 | 13.30 |  |  |  |  |  |  |  |
| 13.30 | 14.00 |  |  |  |  |  |  |  |
| 14.00 | 14.30 |  |  |  |  |  |  |  |
| 14.30 | 15.00 |  |  |  |  |  |  |  |
| etc | etc |  |  |  |  |  |  |  |
| 05.00 | 05.30 |  |  |  |  |  |  |  |
| 05.30 | 06.00 |  |  |  |  |  |  |  |

@@

## Participating Supplier Identification@@

| Supplier | Name |
| --- | --- |
| PS1 |  |
| PS2 |  |
| PS3 |  |
| (etc) |  |

@@

PART 2: TRANSFER UNIT NOMINATION

1.	Name of Participating Generator

2.	Name of Generating Unit

3.	Date on which Trading Day commences

4.	Transfer Unit Nomination (SDC1.4.2.3C (d))

@@

| Settlement Period |  |  | Transfer Unit Nomination |  |
| --- | --- | --- | --- | --- |
|  |  |  | (Totals) |  |
| Start | End |  | (kWh) |  |
| 06.00 | 06.30 |  |  |  |
| 06.30 | 07.00 |  |  |  |
| 07.00 | 07.30 |  |  |  |
| 07.30 | 08.00 |  |  |  |
| 08.00 | 08.30 |  |  |  |
| 08.30 | 09.00 |  |  |  |
| 09.00 | 09.30 |  |  |  |
| 09.30 | 10.00 |  |  |  |
| 10.00 | 10.30 |  |  |  |
| 10.30 | 11.00 |  |  |  |
| 11.00 | 11.30 |  |  |  |
| 11.30 | 12.00 |  |  |  |
| 12.00 | 12.30 |  |  |  |
| 12.30 | 13.00 |  |  |  |
| 13.00 | 13.30 |  |  |  |
| 13.30 | 14.00 |  |  |  |
| 14.00 | 14.30 |  |  |  |
| 14.30 | 15.00 |  |  |  |
| etc | etc |  |  |  |
| 05.00 | 05.30 |  |  |  |
| 05.30 | 06.00 |  |  |  |

@@

PART 3: SUPPLEMENTAL ENERGY BIDS

1.	Name of Participating Generator

2.	Name of Generating Unit

3.	Date on which Trading Day commences

4.	Supplemental Energy Bid for Trading Day

Start Up Price (£)

Fixed Price (£/hour)

Incremental Price (£/MWh)

@@

SDC1 - Appendix H

FACSIMILE

TO	:	Northern Ireland Electricity plc,  Castlereagh House Control Centre             Page 1 of 3

FAX	:

FROM	:

[               ] POWER STATION - DAILY SCHEDULING NOTICES

_____________________________________________________________________________________________________________________

1.	AVAILABILITY NOTICE

The availability of each CD CCGT Installation at [             ] Power Station for the Schedule Day to which this notice refers is shown on page 2 attached.

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

2.	ENERGY BID PRICE NOTICE   (£x.xxxx/GJ - enter bid price where appropriate)

[           Power Limited] offers NIE the following energy prices (which are lower than the contracted prices) for the Schedule Day to which this notice refers.

[Fuel]				[Fuel]

[Fuel]				[Fuel]

[Fuel]

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

3.	GSDP NOTICE

The Generation Scheduling and Despatch Parameters for each CD CCGT Installation at [       ] Power Station for the Schedule Day to which this notice refers are as given in schedule 1 of the relevant Generating Unit Agreement

YES / NO  (Delete as appropriate)

(If NO, each GSDP which varies from schedule 1 must be shown on page 3 attached.

If YES, page 3 shall be omitted)

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

4.	GENERATION OTHER RELEVANT DATA

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

________________________________________________________________________________________________________________

5.	DECLARATION

These notices are issued by [           Power Limited] in respect of the CD CCGT Installation(s)

at [             ] Power Station in accordance with the requirements of SDC1.

We declare that the figures and other information given in the notices fully reflect the actual capability of the CD CCGT Installation(s).

The notices are applicable to Schedule Day:

_______________________________________________________________________________________________________________

Signature:________________________Name:___________________________________________________________________________

(in block capitals)

Position: Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue:

________________________________________________________________________________________________________________

AVAILABILITY NOTICE				Page 2 of 3@@

Settlement 			[UNITS ON GAS]			[UNITS ON BACK-UP FUEL]

Period

Contracted			   1		   2		   1		   2

Capacity

0000 - 0030		_____		_____		_____		_____

0030 - 0100		_____		_____		_____		_____

0100 - 0130		_____		_____		_____		_____

0130 - 0200		_____		_____		_____		_____

0200 - 0230		_____		_____		_____		_____

0230 - 0300		_____		_____		_____		_____

0300 - 0330		_____		_____		_____		_____

0330 - 0400		_____		_____		_____		_____

0400 - 0430		_____		_____		_____		_____

0430 - 0500		_____		_____		_____		_____

0500 - 0530		_____		_____		_____		_____

0530 - 0600		_____		_____		_____		_____

0600 - 0630		_____		_____		_____		_____

0630 - 0700		_____		_____		_____		_____

0700 - 0730		_____		_____		_____		_____

0730 - 0800		_____		_____		_____		_____

0800 - 0830		_____		_____		_____		_____

0830 - 0900		_____		_____		_____		_____

0900 - 0930		_____		_____		_____		_____

0930 - 1000		_____		_____		_____		_____

1000 - 1030		_____		_____		_____		_____

1030 - 1100		_____		_____		_____		_____

1100 - 1130		_____		_____		_____		_____

1130 - 1200		_____		_____		_____		_____

1200 - 1230		_____		_____		_____		_____

1230 - 1300		_____		_____		_____		_____

1300 - 1330		_____		_____		_____		_____

1330 - 1400		_____		_____		_____		_____

1400 - 1430		_____		_____		_____		_____

1430 - 1500		_____		_____		_____		_____

1500 - 1530		_____		_____		_____		_____

1530 - 1600		_____		_____		_____		_____

1600 - 1630		_____		_____		_____		_____

1630 - 1700		_____		_____		_____		_____

1700 - 1730		_____		_____		_____		_____

1730 - 1800		_____		_____		_____		_____

1800 - 1830		_____		_____		_____		_____

1830 - 1900		_____		_____		_____		_____

1900 - 1930		_____		_____		_____		_____

1930 - 2000		_____		_____		_____		_____

2000 - 2030		_____		_____		_____		_____

2030 - 2100		_____		_____		_____		_____

2100 - 2130		_____		_____		_____		_____

2130 - 2200		_____		_____		_____		_____

2200 - 2230		_____		_____		_____		_____

2230 - 2300		_____		_____		_____		_____

2300 - 2330		_____		_____		_____		_____

2330 - 2400		_____		_____		_____		_____

_____________________________________________________________________________________________________________________

Notes

1.	All availabilities shall be expressed in MW.  Alongside the MW figure shall be placed the unit operating status (in brackets) as defined by the Final Outage Programme: 1 = Normal Availability, 2 = Inflexible Outage, 3 = Flexible Planned Outage, 4 = Advanced Flexible Planned Outage, 5 = Deferred Flexible Planned Outage, 6 = Short Term Planned Maintenance Outage, 7 = Return to Service, 8 = Notified Unplanned Outage, 9 = Forced Outage.

2.	For each CD CCGT Installation an availability figure must be entered for the first Settlement Period.  Where the CD CCGT Installation is completely unavailable, a zero shall be entered.  Thereafter, an availability figure shall only be entered where the availability for the CD CCGT Installation is changed from the previously expressed value.

3.	This Availability Notice shall include all planned outages agreed with NIE and all unplanned/forced outages already notified to NIE.  It shall not include unplanned/forced outages not yet notified to NIE unless the appropriate Outage Notice is attached.

_____________________________________________________________________________________________________________________

CD CCGT Installation Matrix showing the combination of CCGT Modules running in relation to any given MW output, in the form of the diagram illustrated below.  The CD CCGT Installation Matrix is designed to achieve certaintly in knowing the number of CCGT Modules synchronised to achieve a Despatch instruction.

CD CCGT Installation Matrix example form

| CD CCGT INSTALLATION ACTIVE POWER | CCGT MODULES AVAILABLE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1st GT | 2nd GT | 3rd GT | 4th GT | 5th GT | 6th GT | 1st ST | 2nd ST | 3rd ST |
| MW | ACTIVE POWER OUTPUT |  |  |  |  |  |  |  |  |
|  | 150 | 150 | 150 |  |  |  | 100 |  |  |
| [ ] MW to [ ] MW | / |  |  |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / |  |  |  |  |  | / |  |  |
| [ ] MW to [ ] MW | / | / |  |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / | / |  |  |  |  | / |  |  |
| [ ] MW to [ ] MW | / | / | / |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / | / | / |  |  |  | / |  |  |

@@

In the absence of the correct submission of a CD CCGT Installation Matrix the last submitted (or deemed submitted) CD CCGT Installation Matrix shall be taken to be the CD CCGT Installation Matrix submitted hereunder.

_____________________________________________________________________________________________________________________

This notice is applicable to Schedule Day: ________________________________________________

Signature: ___________________________________________________

_____________________________________________________________________________________________________________________

GSDP NOTICE				Page 3 of

_____________________________________________________________________________________________________________________

[         Power Limited] declares that the undermentioned CD CCGT Installation(s) are presently unable to perform to the characteristics stated in [schedule 1 of the relevant Generating Unit Agreement] and that the affected characteristics are mentioned below with revised values that should be used for the purposes of scheduling and despatch.

GENERATION SCHEDULING AND 	CONTRACTED	REVISED	      TIME

UNIT	DESPATCH PARAMETER AFFECTED	VALUE	VALUE	FROM	TO

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

____	__________________________________	____________	_________	_____	_____

Examples

1.	Governor Droop	4.0%	4.5%	0000	2400

2.	Loading Rate after Hot Start 30+ MW	6.0	2.5	1800	2200

_____________________________________________________________________________________________________________________

This notice is applicable to Schedule Day: ___________________________________________________________

Signature: __________________________________________________

_____________________________________________________________________________________________________________________

SDC1 - Appendix I

NOTIFICATION OF REVISED AVAILABILITY @@

Settlement 			UNITS ON GAS			UNITS ON BACK-UP FUEL

Period

Contracted			   1		   2		   1		   2

Capacity

0000 - 0030		_____		_____		_____		_____

0030 - 0100		_____		_____		_____		_____

0100 - 0130		_____		_____		_____		_____

0130 - 0200		_____		_____		_____		_____

0200 - 0230		_____		_____		_____		_____

0230 - 0300		_____		_____		_____		_____

0300 - 0330		_____		_____		_____		_____

0330 - 0400		_____		_____		_____		_____

0400 - 0430		_____		_____		_____		_____

0430 - 0500		_____		_____		_____		_____

0500 - 0530		_____		_____		_____		_____

0530 - 0600		_____		_____		_____		_____

0600 - 0630		_____		_____		_____		_____

0630 - 0700		_____		_____		_____		_____

0700 - 0730		_____		_____		_____		_____

0730 - 0800		_____		_____		_____		_____

0800 - 0830		_____		_____		_____		_____

0830 - 0900		_____		_____		_____		_____

0900 - 0930		_____		_____		_____		_____

0930 - 1000		_____		_____		_____		_____

1000 - 1030		_____		_____		_____		_____

1030 - 1100		_____		_____		_____		_____

1100 - 1130		_____		_____		_____		_____

1130 - 1200		_____		_____		_____		_____

1200 - 1230		_____		_____		_____		_____

1230 - 1300		_____		_____		_____		_____

1300 - 1330		_____		_____		_____		_____

1330 - 1400		_____		_____		_____		_____

1400 - 1430		_____		_____		_____		_____

1430 - 1500		_____		_____		_____		_____

1500 - 1530		_____		_____		_____		_____

1530 - 1600		_____		_____		_____		_____

1600 - 1630		_____		_____		_____		_____

1630 - 1700		_____		_____		_____		_____

1700 - 1730		_____		_____		_____		_____

1730 - 1800		_____		_____		_____		_____

1800 - 1830		_____		_____		_____		_____

1830 - 1900		_____		_____		_____		_____

1900 - 1930		_____		_____		_____		_____

1930 - 2000		_____		_____		_____		_____

2000 - 2030		_____		_____		_____		_____

2030 - 2100		_____		_____		_____		_____

2100 - 2130		_____		_____		_____		_____

2130 - 2200		_____		_____		_____		_____

2200 - 2230		_____		_____		_____		_____

2230 - 2300		_____		_____		_____		_____

2300 - 2330		_____		_____		_____		_____

2330 - 2400		_____		_____		_____		_____

@@____________________________________________________________________________________________________________

Notes

1.	All Availabilities shall be expressed in MW.  Alongside the MW figure shall be placed the unit operating status (in brackets) as defined by the Final Outage Programme: 1 = Normal Availability, 2 = Inflexible Planned Outage, 3 = Flexible Planned Outage, 4 = Advanced Flexible Planned Outage, 5 = Deferred Flexible Planned Outage, 6 = Short Term Planned Maintenance Outage, 7 = Return to Service.

2.	For each CD CCGT Installation, an Availability figure shall be entered in respect of the relevant Settlement Period(s) where the Availability of the CD CCGT Installation is changed from a value notified previously in an Availability Notice. Where the change in Availability occurs otherwise than at the commencement of an Availability Period, an appropriate annotation must be made, stating the exact time of the change.  If the CD CCGT Installation is Unavailable, a zero shall be entered.

3.	This Availability Notice shall include all planned outages agreed with NIE and all unplanned/forced outages already notified to NIE.  It shall not include unplanned/forced outages not yet notified to NIE unless the appropriate Outage Notice is attached.

_______________________________________________________________________________________________________

CD CCGT Installation Matrix showing the combination of CCGT Modules running in relation to any given MW output, in the form of the diagram illustrated below.  The CD CCGT Installation Matrix is designed to achieve certaintly in knowing the number of CCGT Modules synchronised to achieve a Despatch instruction.

CD CCGT Installation Matrix example form

@@

| CD CCGT INSTALLATION ACTIVE POWER | CCGT MODULES AVAILABLE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1st GT | 2nd GT | 3rd GT | 4th GT | 5th GT | 6th GT | 1st ST | 2nd ST | 3rd ST |
| MW | ACTIVE POWER OUTPUT |  |  |  |  |  |  |  |  |
|  | 150 | 150 | 150 |  |  |  | 100 |  |  |
| [ ] MW to [ ] MW | / |  |  |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / |  |  |  |  |  | / |  |  |
| [ ] MW to [ ] MW | / | / |  |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / | / |  |  |  |  | / |  |  |
| [ ] MW to [ ] MW | / | / | / |  |  |  |  |  |  |
| [ ] MW to [ ] MW | / | / | / |  |  |  | / |  |  |

In the absence of the correct submission of a CD CCGT Installation Matrix the last submitted (or deemed submitted) CD CCGT Installation Matrix shall be taken to be the CD CCGT Installation Matrix submitted hereunder.

@@____________________________________________________________________________________________________________

This notice is applicable to Schedule Day: ____________________________________________________________________

Signature: ___________________________________________________

SDC1 - Appendix J

FACSIMILE				Page 1 of

TO/FROM:	Northern Ireland Electricity plc, Castlereagh House Control Centre

FAX:		[		]

FROM/TO:	[                   Power Limited]

GSDP REVISION NOTICE

________________________________________________________________________________________________________

1.	[          ] POWER STATION REQUEST

[       Power Limited] advises NIE that the undermentioned GSDPs for the following CD CCGT Installations are not in accordance with

schedule 1 of the relevant GUAs and, as a result, the CD CCGT Installations will be unable to meet their Spinning Reserve Capability:

ESTIMATED

CONTRACTED	REVISED	      START		DURATION

UNIT ID	GSDP AFFECTED	  VALUE   	 VALUE 	DATE	TIME	   HRS

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

_______	________________	__________	_______	_____	_____	_________

Reason for Change

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

Signature: _______________________________________	Name: ________________________________________

(in block capitals)

Position: Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue: ______________________________________

________________________________________________________________________________________________________

2.	NIE RESPONSE TO REQUEST (Delete as applicable)

1.  Request accepted and confirmed as above.

2.  Amendment proposed as shown below:

REVISED VALUE:  _____________________    REVISED ESTIMATED DURATION HRS: _________________

REVISED START DATE: _________________    REVISED START TIME:   ___________________________

Signature: ________________________________    Name: ___________________________________________

(in block capitals)

Position:  Commercial Engineer/Control Engineer (delete as applicable)

Date/Time of Issue: __________________________________

________________________________________________________________________________________________________